POWER EQUIPMENT                                                                                                                               Circuit Breakers. A circuit breaker is a manually or automatically operated electrical switch designed to protect and control power systems by interrupting fault currents.                                                                                                                                                        A circuit breaker is based on the interruption of the electric current in an electrical circuit. A circuit breaker mainly consists of fixed contacts and moving contacts. Unlike fuses, circuit breakers can be reset and reused, making them more sustainable and cost-effective.

Working Principle

- Detection of Fault: The circuit breaker detects a fault condition, such as an overload or a short circuit in the electrical system.

- Activation of Mechanism: Once a fault is detected, the circuit breaker activates a mechanism that triggers the release of stored potential energy.

- Separation of Contacts: This energy then separates the two main contacts of the circuit breaker: a fixed contact and a moving contact.

- Arc Formation: The separation of contacts creates a highly conductive and ionized plasma known as an arc.

- Arc Quenching: The circuit breaker uses various methods to extinguish or quench the arc, such as cooling, compressing, or replacing the ionized medium with fresh gas.

- Current Interruption: This process interrupts the flow of current, stopping any damage from the fault condition.

- Resetting: Once the fault is cleared, the circuit breaker can be manually or automatically reset, allowing the current to flow again.

- Re-establishment of Contacts: Resetting the breaker brings the fixed and moving contacts back together, re-establishing the flow of current through the circuits

TYPES OF CB                                                                                                                                              (i) Oil circuit breaker (ii) Air circuit breaker (iii) SF6 circuit breaker (iv) Vacuum circuit breaker

Oil Circuit Breaker                                                                                                                                      An oil circuit breaker uses mineral oil as the arc quenching medium. The oil has high dielectric strength and good cooling properties. The contacts are immersed in an oil-filled chamber, which also acts as an insulator. When the contacts open, the arc is drawn in the oil and decomposes it into hydrogen gas and other products. The gas bubbles surround the arc and reduce its temperature and pressure. The oil also acts as a barrier to prevent re-striking of the arc.

Oil circuit breakers are suitable for high-voltage applications, such as transmission lines and substations. They have low maintenance costs and long service life.  However, oil circuit breakers have some risks that need to be managed, including:

- Oil leakage and fire risk

- Oil contamination and deterioration

- Large size and weight

- Environmental concerns

### Air Circuit Breaker

An air circuit breaker uses compressed air as the arc quenching medium. The air has low dielectric strength but high thermal conductivity. The contacts are enclosed in an air-tight chamber, which is connected to an air compressor.

When the contacts open, the air blast is directed toward the arc and blows it away from the contacts. The air also cools down the contacts and removes the ionized particles from the gap.  Air circuit breakers are suitable for low-voltage applications, such as distribution panels and industrial circuits. They have a high speed of operation and low arc energy. However, they also have some disadvantages, such as:

- High noise level

- High operating pressure

- Sensitivity to voltage fluctuations

- High cost of installation and maintenance

### SF6 Circuit Breaker

An SF6 circuit breaker uses sulfur hexafluoride gas as the arc quenching medium. The SF6 gas has high dielectric strength and excellent arc extinguishing properties. The contacts are housed in a sealed chamber filled with SF6 gas at low pressure.

When the contacts open, the gas flows through a nozzle and strikes the arc. The gas absorbs the free electrons in the arc and forms negative ions. This reduces the conductivity of the arc and increases its resistance. The gas also cools down the contacts and prevents re-striking.

SF6 circuit breakers are suitable for medium and high-voltage applications, such as power plants and switchyards. They have low maintenance requirements and long service life. However, they also have some disadvantages, such as:

- High cost of gas

- Environmental impact of gas leakage

- Difficulty in the disposal of gas

- Special handling and safety precautions

### Vacuum Circuit Breaker

A vacuum circuit breaker uses a vacuum as the arc quenching medium. The vacuum has very high dielectric strength and very low arc resistance. The contacts are enclosed in a ceramic or metal chamber that is evacuated to very low pressure.

When the contacts open, the arc is initiated in the vacuum and quickly extinguished. The vacuum has very low arc resistance and high dielectric strength, which makes the interruption process very fast and efficient.

Vacuum circuit breakers are suitable for low and medium-voltage applications, such as industrial and commercial circuits. They have low arc energy and noise level, and high reliability and durability. However, they also have some disadvantages, such as:

- High cost of manufacturing and testing

- The limited current rating and breaking capacity

- Difficulty in detecting vacuum loss

- Special care and handling of ceramic or metal chambers

Isolator

An isolator is a type of mechanical switch that is used to electrically isolate electrical circuits from current passing through them. Isolators are used to locally power on and power off machinery when needed for operation, fault finding, or maintenance work. Isolators should not be operated when a machine or system is running and current is flowing through components, they are used when no load is on. Isolators have no internal methods to deal with arcing, for this reason it should be on operation when no current is flowing through.

Isolators are fitted on the incoming electrical supply to your machine, system, or circuit to separate power from the supply and components when needed. This could be when you need to work on the system or when a part or component may need testing or maintenance. Isolators have electrical testing points located on them so users can test for the presence of voltage.

Types of Isolator

- Single break type isolator

- Double break type isolator

- Pantograph isolator

- MCB – Whilst this is not classed purely as an isolator it can be used in certain applications.

| Isolator | Circuit breaker |
| --- | --- |
| An isolator’s primary function is to disconnect an electrical system from its electrical supply. | A circuit breaker is a protection device. They disconnect the system from the supply in the event of an overload or short circuit. They prevent further damage to the equipment by interrupting the power flow. |
| Isolators should only be used during offload. | Circuits’ breakers can be operated during on-load and offload. |
| The breaking capacity of isolators is not very often. | The breaking capacity of circuit breakers is relatively high. |

# A lightning arrester is a device, essentially an air gap between an electric wire and ground, used on electric power transmission systems to protect the insulation and conductors of the system from the damaging effects of lightning. The typical lightning arrester has a high-voltage terminal and a ground terminal. When a lightning surge (or switching surge, which is very similar) travels along the power line to the arrester, the current from the surge is diverted through the arrester, in most cases to earth.

# Lightning Arrester

Definition: The device which is used for the protection of the equipment at the substations against travelling waves, such type of device is called lightning arrester or surge diverter. In other words, lightning arrester diverts the abnormal high voltage to the ground without affecting the continuity of supply. It is connected between the line and earth, i.e., in parallel with the equipment to be protected at the substation.

Types of lightning arresters

- Rod gap arrester

- Horn gap arrester

- Multi-gap arrester

- Expulsion type lightning arrester

- Valve type lightning arrester

## Lightning Arrester vs. Surge Arrester

Lightning Arrester:  As the name suggests, lightning arresters are specifically designed to safeguard against the powerful surges caused by lightning strikes. These strikes induce extremely high voltage spikes that can overwhelm and damage electrical equipment.

Surge Arrester:  Surge arresters take a broader approach, protecting against a variety of transient voltage spikes. These spikes can originate from various sources, including:

Read Potential and current transformers in EEG352.